The Ethernet Debug Communication Link (EDCL)

Overview

The debug support unit can also be connected from through an Ethernet network by using the EDCL. This unit gives the same access to the on-chip AMBA-AHB bus as the standard RS-232 DCL. The details of the DSU and how it is accessed can be found in the DSU section. This section will provide information on how to use and interface to the EDCL.

Hardware

The internal structure of the EDCL unit is shown in figure 1. It consists of an AHB-bus with an Opencores Ethernet MAC, an AHB-RAM and a control-unit connected to it. It has two interfaces: the external interface to a main-AHB bus and a MII interface from the MAC to a PHY. The bus-interface is consists of the ahbmim and ahbmom signals while the MII interface consists of the ethi and etho signals.

Figure 1. The internal structure of the EDCL.

Figure 2 shows the component interface to the EDCL and meaning of the different generics are as follows:

mstndx: 	Selects which master interface index the unit shall have on the AMBA-AHB bus.

macaddrh:   	Sets the most significant 3 B of the Ethernet MAC. It sets the complete 6 B MAC address together with macaddrl.

macaddrl:	Sets the least significant 3 B of the Ethernet MAC.

ipaddrh:	Sets the most significant 2 B of the IP-address. Sets the complete 4 B IP-address together with ipaddrl.

ipaddrl:	Sets the least significant 2 B of the IP-address.

udpport:	Sets the UDP port number.

memtech:	Selects what type of on-chip memory to use. Se the AHB-RAM section for more details.

phyadr:	The address of the physical interface chip to which the EDCL’s Ethernet MAC is connected. This is needed to be able to configure the PHY to the correct operating mode.

phyrstcls:	The number of clock-cycles to wait after reset before the PHY is configured. The PHY chips are usually much slower to recover from a reset than the EDCL and are unavailable during this time. The reset recovery delay can be found in the data-sheet for the PHY and this time must be converted into the number of EDCL clock-cycles it corresponds to. A margin of at least 25 % should be added. It makes the EDCL wait for the reset recovery until attempts to configure the PHY.

And the following applies to the ports:

rstn:	An active-low reset signal should be connected here.

clk:	The main clock

ahbmim:	Master interface in-signals from the AHB-bus.

ahbmom:   	Master interface out-signals to the AHB-bus.

ethi:	In-signals to the MAC in the MII interface.

etho:	Out-signals from the MAC in the MII-interface.

The AHB master-interface port signals are records containing all the AHB-signals. They can be connected directly as records in GRLIB without any knowledge of the individual signals. The ethi and etho records contain all the signals of the MI-interface. They can either be connected to the arbiter (described in the arbiter section) as complete records or to external pins on the FPGA which are connected to a PHY. The signals in the records must be treated individually in the latter case.

component edcl is

generic (

mstndx        : integer := 0;

macaddrh      : integer := 16#00005e#;

macaddrl      : integer := 16#000000#;

ipaddrh       : integer := 16#c0a8#;

ipaddrl       : integer := 16#0035#;

udpport       : integer := 8000;

memtech       : integer := infered;

phyadr        : integer := 0;

phyrstcls     : integer := 100000);

port (

rstn	  : in  std_ulogic;

clk	  : in  std_ulogic;

ahbmim  : in  ahb_mst_in_type;

ahbmom  : out ahb_mst_out_type;

ethi    : in  eth_in_type;

etho    : out eth_out_type

);

end component;

Figure 2. The component interface for the EDCL.

Software

An EDCL backend is provided for GRMON which does all the communication through an Ethernet connection to the EDCL hardware unit. The backend is started using the –eth flag. The user must then also provide an IP-address in dot-format, an UDP-port and a memory-size in kB. These parameters are entered on the command-line. The memory-size is locked at 4 kB at the moment but additional alternatives will become available later.

It is also possible to write own applications to communicate with EDCL hardware. The expected packet format is shown in figure 3. The design uses the IP and UDP protocols from the TCP/IP protocol-suite.

The IP-version used must be four and no options are allowed. Also the type-of-service field must be set to zero which indicates normal service. This also applies to the unused bits in the field. The flags and fragment-offset fields must also be set to zero. The protocol field must be set to 0x11 since UDP is used. The hardware unit does not check neither the header-checksum nor the identification field at reception.

The UDP-port is actually not either checked as the target only provides one service and it is then sufficient to check the IP-address. The UDP-checksum is not checked at reception and is not calculated at all when replying (set to zero).

| Ethernet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| destination address 6 B |  |  |  |  |  | source address 6 B |  |  |  |  |  |  |  |  |  |  | type 2 B |  |  |  | 4–bit version |  |  |  | 4-bit header length |  | 8-bit type of service |  |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 |  |  |  | 10 |  | 11 | 12 |  | 13 |  | 14 |  |  |  |  |  | 15 |  |
| 16-bit total length |  |  |  |  |  | 16-bit identification |  |  |  |  |  |  |  |  |  |  | 3-bit flags |  |  |  | 13-bit fragment offset |  |  |  | 8-bit time to live |  | 8-bit protocol |  |
| 16 |  |  | 17 |  |  | 18 |  |  |  |  |  |  | 19 |  |  |  | 20 |  |  |  | 20,21 |  |  |  | 22 |  | 23 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UDP |  |  |  |  |  |  |  |
| 16-bit header checksum |  |  |  |  |  | 32-bit source address |  |  |  |  |  |  |  |  |  |  | 32-bit destination address |  |  |  | 16-bit source port number |  |  |  | 16-bit destination port number |  | 16-bit UDP length |  |
| 24 |  |  | 25 |  |  | 26 |  | 27 |  |  | 28 |  |  |  | 29 |  | 30 | 31 | 32 | 33 | 34 |  | 35 |  | 36 | 37 | 38 | 39 |
|  |  |  |  |  |  | Application |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16-bit UDP checksum |  |  |  |  |  | offset 2 B |  |  |  |  |  |  |  |  |  |  | 32-bit control word |  |  |  | 32-bit address |  |  |  | data 0-242 words |  |  |  |
| 40 |  |  | 41 |  |  | 42 |  |  |  |  |  | 43 |  |  |  |  | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52-1024 |  |  |  |

Figure 3. The complete packet expected by the EDCL.

The communication can be done with any network API which provides the possibility to fulfil the restrictions above. The hardware interfacing and address resolution will be handled automatically. The EDCL also responds to ARP requests.

The data that must be sent with the UDP/IP packet is the application part shown in figure 3. The offset field is ignored by the target and is only used for aligning the rest of the application words to 4 B word boundaries. The 32-bit address is the address of the first 32-bit word to be read or written on the AHB-bus. The data field must contain the data to be written if a write is to be performed. The lowest address word comes first. If a read was performed the data field in the reply contains the read data. The first word is from the lowest address which is given in the address field. The control-word contains data controlling the transfer and has different interpretations in a transmitted packet and in a reply. The fields in a transmitted packet are shown in figure 4 and the corresponding fields for a reply is found in figure 5.

| 14-bit sequence number | 1-bit read/write | 10-bit length | 7-bit unused |
| --- | --- | --- | --- |

Figure 4. The bitfields of the application-layer control-word in transmitted packet.

| 14-bit sequence number | 1-ACK/NAK | 10-bit length | 7-bit unused |
| --- | --- | --- | --- |

Figure 5. The bitfields of the application-layer control-word in a received packet.

The 14-bit sequence number should be increased for each transmitted packet. The read/write bit is set to one for a write operation and to zero for a read. The length field should contain the amount of data in bytes (the size of the data field in the application field).The 7-bit unused field can be used as wanted and is returned untouched in the reply.

The target has a 14-bit counter which it compares to the sequence number in the received packet. If they match the packet is accepted and the counter is increased. In this case the wanted operation is performed and the result is sent in a reply. The reply will contain the same sequence number and the ACK/NAK bit set to zero which indicates a successful operation. If it was a read the length will contain the amount of data in bytes and the address field contains the address of the first data word (the others are from increasing addresses). If it was a write the length field is set to zero. If the packet was not accepted the ACK/NAK is set to one and the sequence number field contains the expected sequence number. The length field and unused fields contain the same data as in the transmitted packet in this case.